# Abstract

Flood susceptibility maps are judged by how well they rank the ground that floods, yet unlike forecasts, which are scored against persistence, they are rarely compared with the simplest alternative, the record of where floods have already been. This study makes that comparison for infrastructure mapped in OpenStreetMap in Bangladesh, with flood extents mapped from Sentinel-1 radar as labels and reference and every model scored on 10 km blocks held out from training. Each flood year was predicted from the years before it, giving 15 tests in three study regions and four independent validation regions. The record’s lead over a gradient-boosted terrain model fell as the share of a flood reaching ground no earlier flood had reached rose (Spearman ρ=−0.775, p<0.001), whatever the length of the record, and the two were level when about a third of flooded assets stood on such new ground. In Rangpur and Rajshahi, where only 6.8 % of the 2024 river flooding reached new ground, the record ranked it at 0.896 against 0.801 for the model and kept its lead with masks re-mapped from a single satellite, with other training targets and with 40 km blocks, while on the cyclone-struck coast, where most flooding reached new ground, the model led. A global hydrodynamic hazard map was no stronger a rival than the model, and the model given the record as a feature beat both of its parts in 5 of 8 tests, all riverine. Radar-observed extents also made better training labels than terrain thresholds, by 0.101, 0.087 and 0.080 in AUC on held-out areas. The radar masks agreed only moderately with an independent MODIS map, so every score measures agreement with flooding as radar sees it. Where a radar flood history exists, it can serve as a free planning map wherever floods return to the same ground, a susceptibility model earns its place mainly where floods reach new ground, and susceptibility studies should report the record as a baseline and score their models by the share of its ranking error they remove.

Keywords: flood susceptibility; Sentinel-1; spatial cross-validation; temporal validation; gradient boosting; flood record; persistence baseline; Bangladesh

# 1	Introduction

Riverine and flash floods recur across northern Bangladesh in most monsoon seasons and cyclone surges strike its south-west coast, yet the susceptibility maps meant to show where flooding would do most harm are rarely tested against floods that actually occurred. The studies closest to this one report best-model AUCs between 0.87 and 0.98 (Adnan et al., 2023; A. R. Md. T. Islam et al., 2021; R. Islam & Chowdhury, 2024; Talukdar et al., 2020), and two choices decide what such a figure means. The first is where the flood locations come from, usually a few hundred to a few thousand points drawn from field surveys, local knowledge or satellite flood maps. The second is how the test set is separated from the training set, since a random split of spatially clustered points tests a model mostly beside its own training points, a design known to inflate accuracy (Ploton et al., 2020; Roberts et al., 2017).

Both choices can now be made differently, because Sentinel-1 radar sees through monsoon cloud and has been processed into flood maps with overall accuracies above 94 % (Twele et al., 2016; Uddin et al., 2019). Every pixel observed to flood can therefore serve as a training label (R. Islam & Chowdhury, 2024), and whole areas can be held out so that the test asks how well a model carries to ground it has not seen. A radar record of several floods also offers the reference that forecasting takes for granted, since a weather or river forecast is judged by the skill it adds over persistence or climatology (Murphy, 1988; Pappenberger et al., 2015). For a susceptibility map that reference is the map of where water has already been, which susceptibility studies rarely report.

This study asks two questions of flood susceptibility models for mapped infrastructure in three regions with contrasting regimes, riverine flooding in Rangpur and Rajshahi, flash flooding in the Sylhet haor basin and cyclone surge on the south-west coast. The first, and the most consequential for how published maps should be read, is whether a model anticipates a later flood better than the record of earlier ones, and on what the answer depends. It is tested on every flood year that has an earlier one, in the study regions and in four validation regions that share no asset with them, against a global hydrodynamic hazard map as well as the record. The second is whether radar-observed flood extents make better training labels than terrain thresholds for ground a model has not seen. The maps, data and code are released openly.

# 2	Related work

## 2.1	Flood susceptibility in Bangladesh

Table 1 summarises the studies closest to this one, each checked against its full text. They report high accuracies, from 0.81–0.87 for five models in the Teesta basin (A. R. Md. T. Islam et al., 2021) to 0.984 for a random forest on the south-west coast (Adnan et al., 2023), but every study whose split could be read divided its points at random except one, which held out five regions within the 2024 Feni flash flood and found that random splits raised AUC by 0.06–0.09 (Diptho et al., 2026). The inventories also differ in kind, from points identified by field survey and local residents (A. R. Md. T. Islam et al., 2021; Talukdar et al., 2020) or a water-board survey (Rudra & Sarkar, 2023) to pixels sampled from satellite flood maps (Adnan et al., 2023; Diptho et al., 2026; R. Islam & Chowdhury, 2024), so the same accuracy figure can measure agreement with quite different things. Two studies compared the maps their models produced, and those comparisons are the more instructive. On the south-west coast four models each classified roughly 60 % of the land as flood-prone, yet their pixel-wise correlations ranged from 0.62 to 0.91 (Adnan et al., 2023). In one Sylhet sub-district a random forest and a support vector machine with AUCs of 0.964 and 0.954 agreed on only 73.3 % of the map (R. Islam & Chowdhury, 2024). Models that score equally well can therefore disagree about where flooding will happen, which is the property a map exists to convey. A multi-criteria hazard map of the Sangu basin placed 39 % of roads and 25 % of education buildings in its highest hazard class (Zzaman et al., 2021), which shows why infrastructure exposure matters to planners.

## 2.2	Labels and validation

Radar offers training labels that cover every pixel, and Sentinel-1 flood maps have reached overall accuracies of 94.0–96.1 % (Twele et al., 2016) and 96.44 % for the 2017 Bangladesh floods (Uddin et al., 2019). Daily MODIS imagery has mapped 913 large floods worldwide (Tellman et al., 2021), although radar still struggles in vegetated and urban areas (Amitrano et al., 2024), and Sentinel-1 flood maps have already been used to estimate flood exposure across a whole country (Sherpa & Shirzaei, 2022). One Bangladesh study has already built its inventory from Sentinel-1, for a single Sylhet flood and with a random split (R. Islam & Chowdhury, 2024). Where no inventory exists, labels are often thresholded from the topographic wetness index (Beven & Kirkby, 1979) and height above nearest drainage (Nobre et al., 2011), the same indices that serve as predictors, so a model scored against such labels can succeed by relearning the rule that made them.

Random cross-validation of spatially structured data underestimates prediction error, and splitting by area can address this (Roberts et al., 2017). Splitting by time, training on past events and testing on later ones, has long been standard for landslides (Chung & Fabbri, 2003; Guzzetti et al., 2006), but among the Bangladesh flood studies only one map has been tested on later floods, a multi-criteria map of Sylhet scored against the floods of June 2022 and June 2024 (Swarup & Nury, 2026), and none has been set against the record of earlier floods. Where records and models have been compared, it was as hazard maps, with satellite records showing less frequent flooding than global flood models predict (Hawker et al., 2020). Such records already exist for Bangladesh, as a national Sentinel-1 flood-frequency map (Haghighi, 2022), a weekly inundation series built for flood insurance (Thomas et al., 2023) and a two-decade reconstruction (Giezendanner et al., 2023). Blocking can force a model to extrapolate (Roberts et al., 2017) and has no theoretical basis for estimating map accuracy (Wadoux et al., 2021), so blocked scores are read here as performance on unseen areas of similar terrain, not as design-based map accuracy. Calibration is treated apart from ranking, by isotonic regression (Niculescu-Mizil & Caruana, 2005; Zadrozny & Elkan, 2002), and among the Bangladesh studies only Diptho et al. (2026) calibrated its scores, within a single flood event.

## 2.3	Past events as the reference

Other hazard fields score their models against the record of past events as a matter of course. In earthquake forecasting the reference is relative intensity, a map built only from smoothed past seismicity, and in a ten-year experiment in California neither a pattern-recognition model nor the United States national seismic hazard map gained significantly over it (Zechar & Jordan, 2008). That test scores each forecast by its Molchan trajectory, the share of later events missed against the share of space flagged, summarised as an area skill score. Crime analysts likewise build hotspot maps from earlier crimes and judge them by the share of later crimes they hold over the share of area they flag (Chainey et al., 2008). For landslides, models trained on a single extreme year failed to hindcast other years while inventories of more than six to eight years hindcast well (Jones et al., 2021), so a test on one later year says little about the next. The skill a forecast earns also depends on the benchmark it is set against, and the most informative benchmark is the one hardest to beat (Pappenberger et al., 2015). For a map of where floods will occur that benchmark is the map of where they already have, and no flood susceptibility study found here has set its model against it.

# 3	Study area and data

Three regions with contrasting flood regimes are studied, riverine flooding in Rangpur and Rajshahi, flash flooding in the Sylhet haor basin and cyclone surge on the south-west coast (Fig. 1a, Table 2), and they share one set of model settings so that results stay comparable. Rangpur and Rajshahi, in the north-west, lie on the floodplains of the Teesta and the Brahmaputra–Jamuna, where monsoon rivers rise above their danger levels and stay there for days to weeks, and all five mapped floods fell between late June and October (Table 4). The Sylhet region holds the haor basin, a low bowl of seasonal wetlands fed by rivers from the hills to the north and east, where flash floods rise quickly and can arrive before the monsoon, as in April 2017, when they struck before the rice harvest. The south-west coast is a delta of tidal channels and polders, embanked islands of farmland, where flooding follows the landfall of a cyclone and the overtopping or breach of embankments rather than the stage of a river. The regions hold 23,016, 13,374 and 29,545 mapped assets, across parts of 21, 10 and 16 districts. Four further regions serve only to validate the comparison with the flood record (Fig. 1a, e–h, Table 4): the Jamuna east bank, the central districts around Dhaka, the Chattogram coast and the west-central districts, where both mapped events were waterlogging after heavy rain rather than river floods. None holds an asset that a study box also holds. Every dataset the pipeline ingests is openly licensed (Table 3). OpenStreetMap coverage is uneven, so an area with few mapped assets shows low exposure whether or not much stands there, and WorldPop’s constrained product carries population only where buildings were detected in imagery.

# 4	Methods

The pipeline runs as a chain of steps, drawn in the Supporting Information (Fig. S1, Section S1) with the settings not given here, and this section follows it in order.

## 4.1	Assets, terrain and features

Assets are fetched from OpenStreetMap in three tag groups, lifelines (hospitals, schools, shelters, bridges, embankments), transport (primary to trunk roads, railways) and agriculture (farmland, aquaculture, canals, markets), and each is classified into one asset type. The configured boxes cross Bangladesh’s land border, so assets in India, or on land more than about 1 km outside the geoBoundaries border, are dropped, while those on chars, coastal ground and offshore are kept. Grid cells and grid-level validation are restricted the same way. From the 30 m SRTM elevation model, reprojected to UTM zone 46N, the pipeline derives slope, the topographic wetness index (TWI), height above nearest drainage (HAND) and D8 flow accumulation. FABDEM, which removes forest and building heights (Hawker et al., 2022), would suit a flat delta better, but its non-commercial, share-alike licence conflicts with the CC BY 4.0 data archive, and the vertical error of SRTM may be one reason, untested, why TWI and HAND add little once elevation is known (Section 5.6). Each asset carries these five terrain values at its representative point, WorldPop population density, its distance to the nearest hospital, school, flood shelter, road and irrigation channel, and its asset type, all standardised, and the pipeline stops if any feature is constant or the labels hold a single class.

## 4.2	Flood extents from Sentinel-1

Flood extents mapped from Sentinel-1 radar supply both the training labels and the reference against which the models are scored. Following the change-detection method recommended by UN-SPIDER (UN-SPIDER, 2019), an approach whose best Sentinel-1 variant reached an overall accuracy of 86.3 % against reference flood maps in an evaluation for Thailand (Dadhich et al., 2026), the minimum VV backscatter in each flood window is compared in Google Earth Engine with the median of a dry-season baseline from the same year. A pixel is flagged as flooded where it is below −16 dB and at least 3 dB darker than its baseline. Permanent water from the JRC record (Pekel et al., 2016), slopes above 5​∘ and specks under eight pixels are removed, and the event masks are averaged into a flood-frequency layer (Fig. 1b–d). Five events are mapped in Rangpur and Rajshahi, four in Sylhet and four cyclone landfalls on the coast, each window set from ReliefWeb situation reports (Table 4).

The masks were checked against an independent map of two of the same floods, the August 2017 flood in Rangpur and Rajshahi and the April 2017 haor flood in Sylhet, which the Global Flood Database maps from MODIS at 250 m (Tellman et al., 2021). On that grid, over cells that MODIS saw clear and that are not permanent water, the maps agree on 81.9 % and 82.0 % of cells, a figure dominated by dry ground. On flooded ground the agreement is only moderate, with a critical success index of 0.290 and 0.339 and a kappa of 0.346 and 0.406. The radar marks more ground as flooded than MODIS in Rangpur and Rajshahi (19.7 % against 13.2 %), consistent with flooded patches smaller than a MODIS pixel and with flooding hidden by cloud. It marks less in the Sylhet haor (13.2 % against 23.3 %), consistent with the difficulty radar has where water stands under vegetation (Amitrano et al., 2024), as it did under unharvested rice in April 2017. Neither explanation was tested, and neither map is ground truth.

## 4.3	Training labels

In the riverine regions ground counts as flood-prone where it flooded in at least two of the mapped events, while on the coast one cyclone window is enough, because surge drains within days and the six-to-twelve-day revisit leaves too few pixels flooded twice. The pipeline also writes a terrain-threshold ensemble, which marks a pixel as flood-prone where at least two of three indicators agree, TWI above 8, HAND below 5 m and JRC water occurrence above 25 %. TWI and HAND are also model inputs, so a model trained and scored on these labels can succeed by relearning the threshold rule.

## 4.4	Asset model

The default asset model is class-weighted gradient boosting with 300 iterations and a learning rate of 0.1, which matched or beat a random forest, logistic regression and a two-layer GraphSAGE network (Hamilton et al., 2017) on a six-nearest-neighbour graph of assets, the design this system began with (Supporting Information, Section S7). Validation holds out whole 10 km blocks, assigned at random to training (80 %), calibration and test, because a random split leaves every test asset beside training assets with almost the same features and label. Scores are calibrated by isotonic regression on the calibration blocks, with each level shrunk towards the middle when it holds few points, and every metric comes from the test blocks. In the two riverine regions the published map scores come from the same model given one more feature, the share of earlier events in which the ground flooded (Section 5.3). Every validation figure outside that section comes from the model without it, because the feature is built from the events the models are scored against.

## 4.5	Validation design

Every model is scored on the held-out blocks by the area under the ROC curve (AUC), average precision and its lift over the base rate, and the Brier score. Every comparison between models or label sources is repeated over 20 block assignments drawn with different seeds. The paired differences are tested by the corrected resampled t-test of Nadeau & Bengio (2003), since the assignments reuse the same assets, and a plain t-test across them would overstate the evidence. The 10 km blocks are narrower than the ranges of the variograms fitted to the asset scores, roughly 115, 14 and 31 km, so test assets near a block edge still share terrain with training assets, and the main results are repeated with 20 and 40 km blocks on the same seeds (Section 5.5).

## 4.6	Later floods and published maps

The model and the record are compared on later floods in two designs. The first trains on the floods up to 2022 and scores against those of 2024 (Section 5.3). The second, a rolling origin, makes every flood year with an earlier event a test in turn. The model learns from the earlier years only, under the region’s labelling rule capped by their number of events, the record is the share of those events in which the ground flooded, and both are scored on the held-out blocks against that year’s floods. Each test also records the share of that year’s flooded assets standing on ground no earlier flood had reached, termed new ground here, the quantity expected to decide which of the two ranks better. Four validation regions join the three study regions in this design, the Jamuna east bank, the central districts around Dhaka, the Chattogram coast and the west-central districts, each a region of a national partition of Bangladesh with every asset that a study box also holds removed, matched by OpenStreetMap identity or by location (Table 2). Distances there run to every facility of the region, including those removed.

Two published maps are scored on the same assets. The JRC global river flood hazard maps (Baugh et al., 2024), simulated with LISFLOOD-FP at about 90 m for return periods of 10 to 500 years, give each asset the annual exceedance probability of the most frequent flood that reaches it. JRC Global Surface Water occurrence (Pekel et al., 2016), the share of Landsat observations from 1984 to 2021 that saw water, stands for a record available anywhere without processing radar and is scored only against floods after 2021. Besides AUC, every ranking is scored by its Molchan trajectory and area skill score (Zechar & Jordan, 2008), with every asset weighted equally, so that a random ranking scores 0.5, and by the prediction accuracy index (Chainey et al., 2008) of a list as long as the record’s, the share of flooded assets the list holds divided by the share of assets it flags.

# 5	Results

## 5.1	Held-out results

Table 5 reports one block assignment, the pipeline’s default, which ranks 2, 1 and 17 of 20 by AUC in the three regions. In Sylhet 14.7 % of assets sit on flood-prone ground and the asset model reaches an AUC of 0.934. In Rangpur and Rajshahi only 6.7 % do, yet the model ranks well (AUC 0.884), with an average precision 7.0 times the base rate. Calibration lowers the Brier score from 0.069 to 0.029 in Rangpur and Rajshahi, slightly below the 0.032 of always predicting the base rate, and its deciles lie close to the diagonal in both riverine regions (Fig. 3). It carries less well to the coast, where the calibration blocks hold 5.5 % flood-prone assets against 1.7 % in the test blocks and the calibrated Brier score of 0.039 is worse than the base rate at 0.017.

## 5.2	Observed extents against terrain labels

The terrain-threshold ensemble captures 0.151 of the ground that flooded in at least 2 of the five Rangpur and Rajshahi events, with a false alarm ratio of 0.517, and marks 6.6 % of the region as flood-prone where the radar saw 21.2 % flood that often. Trained on those labels and scored on the same held-out blocks against observed flooding over 20 assignments, the default model reaches 0.716 against 0.817 for observed labels in Rangpur and Rajshahi, 0.770 against 0.857 in Sylhet and 0.758 against 0.839 on the coast. The gaps of 0.101, 0.087 and 0.080 favour the observed labels on 20, 19 and 19 of the assignments (p<0.001, p=0.003 and p<0.001; Fig. 2). This test favours the observed labels, since that model is scored against the definition it learned, so Section 5.3 repeats it against later floods that neither label source was trained on.

## 5.3	Transfer to later floods

The held-out blocks separate training from testing in space but not in time. Each model is therefore also trained on the floods up to 2022, keeping each region’s labelling rule capped by the number of earlier events, and scored on the held-out blocks against the ground any 2024 flood reached. Three references are scored on the same assets over 20 assignments (Table 6). The first is the share of earlier events in which the ground flooded, which is the map the radar record alone provides. The second is the same model trained with the 2024 events included, and the third is the same model trained on terrain-threshold labels, which draw on no flood event.

Trained only on earlier floods, the model ranks the 2024 flooding at 0.801 in Rangpur and Rajshahi, 0.797 in Sylhet and 0.807 on the coast. In both riverine regions the record of earlier flooding ranks it better, at 0.896 and 0.851, with the model ahead on only 2 and 4 of 20 assignments, whereas on the coast the model leads, at 0.807 against 0.668. The regions differ in how often floods return to the same ground, since only 6.8 % and 22.1 % of the assets flooded in 2024 stood on ground no earlier event had reached, against 62.9 % on the coast, where each surge floods the stretch of coast where it lands. The 2024 floods in Rangpur and Rajshahi were also small, covering 4.9–8.3 % of the region against 19.9–24.8 % for each earlier flood, whereas in Sylhet they were as large as earlier ones.

Seven checks test whether the comparison favours one side (Supporting Information, Section S5, Table S1), and in Rangpur and Rajshahi the record keeps its lead under all of them, if narrowly under two. Re-mapped from Sentinel-1A alone, since Sentinel-1B failed in December 2021 and fewer scenes can map less flooding, the earlier events shrank by up to 15.7 % and the record still led by 0.097. Retrained on flooding in any earlier event, which is how the 2024 floods are counted, the model fell further behind, by 0.133, and with 40 km blocks the lead grew to 0.126. In Sylhet the record’s lead closed, to 0.017, when the model learned the same target as the test, so the Sylhet result rests on the choice of target, and on the coast the model’s lead fell within noise with 20 and 40 km blocks.

East of the Jamuna, where the October 2024 flood reached only 43 assets, the record again led, at 0.881 against 0.713 on held-out blocks, but with a handful of flooded assets in most test blocks the lead fell short of significance (p=0.089) except with 40 km blocks (p=0.044). Out of fold over the whole region it led at 0.915 against 0.867, with a block-bootstrap interval for the difference from −0.129 to 0.019, and its list of 6.5 % of assets held 86.0 % of the flooded assets against 48.8 % for the model.

The record can also enter the model as a feature, provided it comes from floods before the ones the model learns to predict, or the model would copy its own labels. The model is therefore trained to predict the last event year up to 2022 from its usual features and the share of the earlier events in which the ground flooded, and scored against the 2024 floods with that share taken over every event to 2022. In Rangpur and Rajshahi the combination beats both of its parts, at 0.924 against 0.896 for the record alone (p=0.020) and 0.790 for the model without the feature. Its gain comes on the ground no earlier flood reached, 87.7 % of the test assets, where the record gives every asset a score of zero and the combined model still ranks the 2024 flooding at 0.729. In Sylhet the combination, at 0.849, cannot be told apart from the model without the feature (p=0.196) or the record alone (p=0.881). On the coast, where the design leaves a single cyclone to train on, the combination at 0.648 cannot be told apart from the record alone (p=0.580).

Measured as skill over the record (Supporting Information, Section S6), the share of the record’s remaining ranking error that a model removes, the model scored −0.905 in Rangpur and Rajshahi, −0.364 in Sylhet and 0.416 on the coast, and 0.276 in Rangpur and Rajshahi given the record as a feature. A list of every asset an earlier flood reached held 93.2 % of the assets the 2024 floods reached in Rangpur and Rajshahi against 68.5 % for an equally long list from the model. On the coast, where the model ranked better overall, the record’s list of 2.5 % of assets still held 37.1 % against 9.5 %, marking the most exposed ground more precisely even where it ranked the rest worse.

The 2024 floods also give the label comparison a test that favours neither label source. The observed labels still lead, at 0.801 against 0.732 in Rangpur and Rajshahi, 0.797 against 0.773 in Sylhet and 0.807 against 0.736 on the coast, ahead on 18, 15 and 18 of 20 assignments. The coastal gap of 0.071 stays close to its spatial 0.080, while the riverine gaps shrink, from 0.101 to 0.069 in Rangpur and Rajshahi and from 0.087 to 0.024 in Sylhet, where the remaining gap cannot be told apart from noise. Much of the Sylhet advantage in the spatial test therefore came from the shared definition rather than from the labels themselves.

## 5.4	Every flood year

Predicting every flood year from the years before it gives 15 tests, 8 in the study regions and 7 in the validation regions (Table 7). The record ranked the later flooding significantly better in 2, the model in 3, and in the other 10 the two could not be told apart, so the 2024 result in Rangpur and Rajshahi is not the rule for every year. The share of new ground decided which way a test went, ranging from 6.8 to 76.1 % (Fig. 4a). The record’s lead fell as that share rose (Spearman ρ=−0.775, p<0.001), in the study regions (ρ=−0.833) and the validation regions (ρ=−0.679, p=0.094) alike, and the fitted line crosses zero at 33.4 % (95 % interval 18.2–41.1 %). New ground shrinks as the record lengthens, but the relation held with the number of earlier years held fixed (partial ρ=−0.705, p=0.003), while that number added nothing once new ground was known (ρ=0.140), and the relation was unchanged with the size of each flood held fixed (ρ=−0.785). The relation also held with each region left out in turn and within every region with more than one test, although resampling whole regions widened the crossover interval to 2.6–42.6 % (Section S9). In Rangpur and Rajshahi the model ranked the 2019 flood better, when 47.9 % of it reached new ground, the two were level in 2020, and the record led in 2024, at 6.8 %. Given the record as a feature, the model ranked the later flooding above both of its parts in 5 of 8 tests, all riverine.

The JRC hazard map ranked the later floods at 0.418 to 0.876, below chance on the coast, whose surges it does not simulate, so it is a fair rival only for river flooding. It trailed the record significantly in 5 tests and led it in 4, each on a flood that reached much new ground, and the model ranked above it in 8 of 15. Landsat water occurrence, scored on the 8 floods after 2021, ranked them at 0.624 to 0.834, below the radar record in 4. Area skill scores ordered the record and the model as AUC did in 15 of 15 tests, and the Molchan trajectories show where the difference lay (Fig. 4b, c). In Rangpur and Rajshahi the record caught most of the 2024 flooding within the first sixth of the assets it flagged and then, holding nothing on the rest, ran down the diagonal, while on the coast the model missed fewer flooded assets at most list lengths.

## 5.5	Larger blocks

Wider blocks lower the held-out AUC of the default model only a little in the riverine regions, from 0.817 at 10 km to 0.791 at 40 km in Rangpur and Rajshahi and from 0.857 to 0.836 in Sylhet, and more on the coast, from 0.839 to 0.774, so the 10 km figures carry a small optimism, largest on the coast.

## 5.6	What the model relies on

Shuffling each feature across the test assets over the same assignments shows what the default model uses (Fig. 5). Elevation carries almost all of the terrain signal, costing 0.086, 0.220 and 0.129 in AUC when shuffled, while TWI, HAND, slope and flow accumulation cost at most 0.000, 0.009 and 0.007. Shuffled together, since correlated features can stand in for one another, the five terrain features cost 0.080, 0.214 and 0.132, about as much as elevation alone, and slightly less in Rangpur and Rajshahi, a difference within the noise of the shuffling. The five access distances together cost 0.110, 0.133 and 0.142, more than the terrain in Rangpur and Rajshahi and on the coast and second to it in Sylhet. They describe distance to services rather than any property of flooding and most likely stand in for where roads and settlements sit on the floodplain, a reading that was not tested directly.

# 6	Discussion

## 6.1	What the flood record already knows

Whether a susceptibility model anticipates a later flood better than the record of earlier ones depended, across 15 tests in seven regions, on how much of that flood reached new ground. Where less than about a third did, as in Rangpur and Rajshahi and Sylhet in 2024 and east of the Jamuna, the record ranked better or could not be told apart from the model, and where more did, as on the coast and in the first years of each record, the model ranked better in all but one test, significantly in 3. The share of new ground is known only after a flood, but in the riverine regions it fell as the record lengthened, from 47.9 to 6.8 % in Rangpur and Rajshahi and from 43.1 to 14.0 % east of the Jamuna, while on the coast, where each surge strikes its own stretch, it stayed near 62.9 %. It is therefore a property of the flood regime that a record of a few events already reveals rather than an accident of one year. Earthquake forecasting reached the same verdict for a hazard that also recurs where it struck before, with neither a pattern-recognition model nor a national hazard map gaining on past seismicity (Zechar & Jordan, 2008), and the hydrodynamic map scored here, which trailed the record significantly in 5 tests, shows that the comparison bears on hazard maps generally and not only on machine learning. A published multi-criteria map of Sylhet scored on the same 2024 floods reached 0.794 (Swarup & Nury, 2026), close to this model’s 0.797, while the record ranked them at 0.851, a comparison that is only indicative, since that map was scored over areas.

The record marks where water returns and the terrain ranks where it has not yet been seen, and where floods recur the two combine well, the model given the record as a feature beating both in 5 of 8 tests. Floods returning to the same ground are expected on a floodplain, yet in Rangpur and Rajshahi a model scoring 0.817 on held-out blocks still ranked the 2024 flood below that expectation, at a skill over the record of −0.905. Forecasts are scored against persistence in the same way (Murphy, 1988; Pappenberger et al., 2015), and a susceptibility study that reports accuracy without that reference cannot show that its model knows more than the flood record already does.

## 6.2	Labels decide what an accuracy figure means

The label source decides what a published accuracy figure measures, since terrain-threshold labels cost the model 0.101, 0.087 and 0.080 in AUC on held-out blocks. The indices those labels are built on, TWI and HAND, add almost nothing to the model once elevation is known, and a model scored against labels derived from its own inputs measures its recovery of the labelling rule rather than its skill at locating floods. The mean AUCs here, 0.817 and 0.857 in the riverine regions, sit at or below the best-model AUCs of 0.87 to 0.98 in Table 1. The comparison is only indicative, since all but one of those studies split their points at random while every figure here comes from 10 km blocks the model never saw. Blocking has its own critics (Roberts et al., 2017; Wadoux et al., 2021), and the blocked scores estimate performance on unseen areas of similar terrain rather than design-based map accuracy.

## 6.3	Implications for flood risk management

First, where Sentinel-1 has observed several floods, the record of where water has already stood is a benchmark any susceptibility map should beat, and where floods return to the same ground it was the better guide to the next one. It costs little to compute, so agencies such as the Bangladesh Water Development Board and the Flood Forecasting and Warning Centre could publish it as a planning layer beside any susceptibility map. Where the record holds only one or two flood years, or where surges strike new stretches of coast, the model ranked better, and the two combined ranked best on the riverine floodplains, so the choice between them can follow how much of past floods fell on new ground. Second, a susceptibility map used to site shelters, embankments or schools should be judged on held-out areas and later floods rather than on random splits, since the accuracy a random split reports overstates how far the map can be trusted in places it has not seen.

## 6.4	Limits of the maps

The coast is the clearest limit, since the terrain hazard surface reaches only 0.610 there on held-out ground and the river hazard map ranks surge flooding below chance, because surge follows embankments, tidal channels and landfall position, which neither describes. The composite risk index (Supporting Information, Section S2) depends on OpenStreetMap, whose rural completeness is unmeasured, and has not been checked against losses. The radar labels carry error of their own, agreeing only moderately with an independent MODIS map and counting inundated paddy as flooding, so every score here measures agreement with flooding mapped by radar rather than with flooding on the ground. Each test draws on one to three earlier flood years, and the threshold near a third of flooding on new ground is estimated from 15 tests in 7 regions, with an interval of 18.2 to 41.1 % when tests are resampled and 2.6 to 42.6 % when whole regions are. The calibrated probabilities hold for a region as a whole rather than for any one area, and none of the surfaces is a forecast.

# 7	Conclusions

A flood susceptibility model knew more than the flood record only where floods reached new ground. Across 15 tests in seven regions of Bangladesh, the record’s lead over a terrain model fell as the share of each flood reaching new ground rose (ρ=−0.775), and the two were level at about a third of flooded assets. In Rangpur and Rajshahi the record predicted the 2024 flooding better than the model under every check applied, a global hydrodynamic hazard map was no stronger a rival, and a model given the record as a feature beat both of its parts in most riverine tests. Radar-observed flood extents made better training labels than terrain thresholds, by 0.101, 0.087 and 0.080 in AUC on held-out ground and by 0.069, 0.024 and 0.071 against later floods. For flood management, a radar flood history is therefore a free planning map wherever floods return to the same ground, and a model is worth building mainly where floods reach new ground. Every number in this paper is generated from the released pipeline outputs, so each can be checked and rebuilt.

Table 1. Flood susceptibility studies in Bangladesh compared with this study. Every study was read in full, and where a figure is given for training and validation separately the validation figure is shown. This system’s figures are means over 20 assignments of spatially held-out blocks, scored against observed floods, so the columns are not directly comparable, for the reasons given in the text.

| Study | Region | Model | Flood locations and split | Reported accuracy |
| --- | --- | --- | --- | --- |
| Talukdar et al. (2020) | Teesta basin | bagging ensembles | 207 flood, 206 non-flood points from inundation maps, topographic maps and local survey, random 80:20 | AUC 0.945 (BgM5P) |
| A. R. Md. T. Islam et al. (2021) | Teesta basin | Dagging, RS, RF, ANN, SVM | 167 flood, 167 non-flood points from records, fieldwork and Google Earth, random 80:20 | AUC 0.81–0.87 (validation) |
| R. Islam & Chowdhury (2024) | Sylhet, one sub-district | RF, SVM | 1,500 flooded, 1,500 dry pixels from Sentinel-1, one event (June 2022), random 70:30 | AUC 0.964 (RF) |
| Rudra & Sarkar (2023) | Sylhet division | ANN | 1,280 flood and non-flood points from a Bangladesh Water Board survey, random 75:25 | prediction rate 89–92 % |
| Swarup & Nury (2026) | Sylhet division | AHP, 19 indicators | independent flood inventories of June 2022 and June 2024, no training data | AUC 0.836 (2022), 0.794 (2024) |
| Diptho et al. (2026) | Feni basin | LightGBM, XGBoost, RF | 1,510 flooded and dry pixels from the observed August 2024 inundation, five spatial blocks | spatial AUC 0.938 (LightGBM) |
| Adnan et al. (2023) | south-west coast | RF, KNN, MLP, GA-RBF-SVR | 965 points sampled from a satellite flood-frequency map (1988–2012), 70:30 split | AUC 0.984 (RF), inter-model r 0.62–0.91 |
| This study | Rangpur & Rajshahi | Gradient boosting | Sentinel-1, 5 events, 10 km blocks | mean AUC 0.817 |
| This study | Sylhet | Gradient boosting | Sentinel-1, 4 events, 10 km blocks | mean AUC 0.857 |
| This study | south-west coast | Gradient boosting | Sentinel-1, 4 cyclones, 10 km blocks | mean AUC 0.839 |

Table 2. Configured study regions. ​aValidation region, defined by its districts within the box and without any asset a study box holds. Each box is restricted to Bangladesh, and all use UTM zone 46N (EPSG:32646) for distance and area calculations.

| Region | Hazard | Bounding box (W, S, E, N) | Configuration |
| --- | --- | --- | --- |
| Rangpur & Rajshahi | riverine flood | 88.0, 24.0, 89.9, 26.7 | config.yaml |
| Sylhet | flash flood | 90.6, 24.0, 92.7, 25.5 | configs/sylhet.yaml |
| South-west coast | coastal and tidal flooding | 88.9, 21.5, 90.8, 23.2 | configs/sw_coastal.yaml |
| Jamuna east​a | riverine and flash flood | 89.6, 23.9, 90.9, 25.5 | configs/validation/jamuna_east.yaml |
| Central​a | riverine flood | 89.25, 22.8, 91.08, 24.39 | configs/validation/central.yaml |
| Chattogram coast​a | flash flood and surge | 91.26, 20.54, 92.4, 23.04 | configs/validation/chattogram_coast.yaml |
| West-central​a | waterlogging | 88.51, 22.75, 89.86, 24.24 | configs/validation/west_central.yaml |

Table 3. Datasets ingested by the pipeline. JRC is the European Commission Joint Research Centre and GloFAS its Global Flood Awareness System. National files (WorldPop, geoBoundaries, JRC tiles) are downloaded once into a shared cache and clipped per region.

| Dataset | Use | Resolution | Licence |
| --- | --- | --- | --- |
| OpenStreetMap | infrastructure assets (roads, bridges, schools, hospitals, shelters, embankments, cropland) | vector | ODbL 1.0 |
| SRTM 1 arc-second | elevation, slope, topographic wetness, height above drainage, flow accumulation | 30 m | public domain |
| JRC Global Surface Water | historical water occurrence, one proxy-label vote | 30 m | free with attribution |
| JRC global river flood hazard maps v2.1.2 (GloFAS) | reference ranking for later floods | 3 arc-seconds (about 90 m) | CC BY 4.0 |
| WorldPop constrained 2020 | population density for vulnerability and population exposure | 100 m | CC BY 4.0 |
| geoBoundaries gbOpen | districts (ADM2), upazilas (ADM3), unions (ADM4) | vector | CC BY 4.0 |
| Sentinel-1 GRD (via Earth Engine) | observed flood extents for training labels and validation | 10 m, exported at 100 m | Copernicus open |

Table 4. Flood events mapped from Sentinel-1. Windows bracket the flood peak; the baseline for each is the February–March dry season of the same year (mid-January to March for the pre-monsoon haor event). Sources are ReliefWeb and United Nations situation reports and, for the west-central waterlogging, national newspapers, all cited in the configuration files.

| Region | Window | Basis |
| --- | --- | --- |
| Rangpur & Rajshahi | 12–26 Aug 2017 | second monsoon flood from 12 August; Dinajpur, Kurigram, Lalmonirhat and Rangpur among the worst affected |
|  | 12–26 Jul 2019 | rain from 9 July, Brahmaputra–Jamuna rising to 18 July, Kurigram more than 1 m above danger level |
|  | 11–31 Jul 2020 | second phase from 9–10 July, peak about 18 July, 27 basin gauges above danger into late July |
|  | 28 Jun–16 Jul 2024 | northern floods from mid-June, Brahmaputra above danger at Nunkhawa, Hatia and Chilmari in early July |
|  | 27 Sep–12 Oct 2024 | Teesta flooding of Lalmonirhat, Nilphamari, Kurigram and Rangpur from late September |
| Sylhet | 30 Mar–20 Apr 2017 | haor flooding from 28 March, striking Sunamganj on 1 April |
|  | 15–26 Jun 2022 | heavy rain from 15 June, situation worst from 17 June |
|  | 16–28 Jun 2024 | 2.25 million affected in Sylhet, Sunamganj and Moulvibazar by 22 June |
|  | 18–30 Aug 2024 | eastern floods from 20 August, including Moulvibazar and Habiganj |
| South-west coast | 20–31 May 2020 | Cyclone Amphan, landfall 20 May |
|  | 25 May–4 Jun 2021 | Cyclone Yaas, landfall 26 May |
|  | 23 Oct–2 Nov 2022 | Cyclone Sitrang, landfall 24–25 October |
|  | 26 May–5 Jun 2024 | Cyclone Remal, landfall 26–27 May |
| Jamuna east | 12–26 Aug 2017 | second monsoon flood from 12 August |
|  | 13–28 Jul 2019 | Brahmaputra–Jamuna rising to 18 July |
|  | 11–31 Jul 2020 | second phase from 9–10 July, peak about 18 July |
|  | 3–15 Oct 2024 | north-central flash floods from 3 October in Sherpur, Mymensingh and Jamalpur; first Sentinel-1 pass on 8 October |
| Central | 15–31 Aug 2017 | Faridpur, Rajbari, Manikganj and Munshiganj among the affected districts |
|  | 18 Jul–2 Aug 2019 | monsoon flood reaching the central districts by 19 July |
|  | 15 Jul–10 Aug 2020 | second phase of the 2020 monsoon flood |
| Chattogram coast | 5–15 Aug 2023 | flash floods and monsoon rain in Chattogram division |
|  | 26 May–1 Jun 2024 | Cyclone Remal |
|  | 20 Aug–5 Sep 2024 | eastern flash floods |
| West-central | 13–30 Sep 2024 | waterlogging of low-lying Jessore after heavy rain |
|  | 25 Jul–25 Aug 2025 | waterlogging of the Bhabodah area |

Table 5. Validation on held-out 10 km test blocks, for the pipeline’s default block assignment, from the model without the past-flooding feature that the riverine maps carry (Section 4.4). Positive rate is the share of assets on ground the radar saw flood in the required number of events. The Brier score is that of the raw class-weighted scores, and the calibrated probabilities are reported in the text. Means over repeated assignments, lower in the riverine regions and higher on the coast, are in the Supporting Information, Table S3.

| Region | Assets | Positive rate | Asset AUC | Asset AP | Brier | Terrain AUC |
| --- | --- | --- | --- | --- | --- | --- |
| Rangpur & Rajshahi | 23,016 | 6.7 % | 0.884 | 0.234 | 0.069 | 0.621 |
| Sylhet | 13,374 | 14.7 % | 0.934 | 0.639 | 0.073 | 0.788 |
| South-west coast | 29,545 | 4.5 % | 0.794 | 0.044 | 0.134 | 0.757 |

Table 6. AUC against the 2024 flood extents on held-out 10 km test blocks, as mean ± standard deviation over 20 block assignments. The model to 2022 is trained only on the earlier events, the all-events model on labels that include 2024, past flooding is the share of the earlier events in which the ground flooded, and the terrain-label model is the same model trained on the terrain-threshold labels. The 2024 events reached 2.8 %, 17.7 %, 2.9 % and 1.1 % of assets in the four regions, and east of the Jamuna only 13 assignments left flooded assets in the test blocks.

| Region | Model to 2022 | All events | Past flooding | Terrain labels |
| --- | --- | --- | --- | --- |
| Rangpur & Rajshahi | 0.801 ± 0.063 | 0.794 | 0.896 ± 0.042 | 0.732 ± 0.067 |
| Sylhet | 0.797 ± 0.064 | 0.850 | 0.851 ± 0.039 | 0.773 ± 0.043 |
| South-west coast | 0.807 ± 0.055 | 0.834 | 0.668 ± 0.072 | 0.736 ± 0.066 |
| Jamuna east (validation) | 0.713 ± 0.243 | 0.720 | 0.881 ± 0.192 | 0.619 ± 0.184 |

Table 7. Every flood year predicted from the years before it, as mean AUC on held-out 10 km blocks over 20 block assignments. New ground is the share of that year’s flooded assets standing on ground no earlier flood had reached. The model given the record as a feature needs two earlier years, and the water occurrence ends in 2021, so it is scored only on later floods. The p-value is the corrected resampled t-test of model against record. ​aValidation region.

|  |  | Earlier | New |  |  | Model | Hazard | Water |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Region | Year | years | ground (%) | Record | Model | + record | map | occurrence | p |
| Rangpur & Rajshahi | 2019 | 1 | 47.9 | 0.703 | 0.737 | – | 0.750 | – | 0.237 |
| Rangpur & Rajshahi | 2020 | 2 | 31.3 | 0.790 | 0.783 | 0.810 | 0.719 | – | 0.721 |
| Rangpur & Rajshahi | 2024 | 3 | 6.8 | 0.896 | 0.801 | 0.924 | 0.807 | 0.785 | 0.009 |
| Sylhet | 2022 | 1 | 76.1 | 0.601 | 0.696 | – | 0.729 | 0.708 | 0.009 |
| Sylhet | 2024 | 2 | 22.1 | 0.851 | 0.797 | 0.849 | 0.785 | 0.768 | 0.036 |
| South-west coast | 2021 | 1 | 30.8 | 0.788 | 0.822 | – | 0.434 | – | 0.710 |
| South-west coast | 2022 | 2 | 65.4 | 0.679 | 0.794 | 0.650 | 0.485 | 0.799 | 0.134 |
| South-west coast | 2024 | 3 | 62.9 | 0.668 | 0.807 | 0.648 | 0.418 | 0.624 | <0.001 |
| Jamuna east​a | 2019 | 1 | 43.1 | 0.739 | 0.735 | – | 0.876 | – | 0.955 |
| Jamuna east​a | 2020 | 2 | 28.5 | 0.795 | 0.742 | 0.836 | 0.820 | – | 0.179 |
| Jamuna east​a | 2024 | 3 | 14.0 | 0.881 | 0.713 | 0.920 | 0.872 | 0.834 | 0.089 |
| Central​a | 2019 | 1 | 41.3 | 0.718 | 0.882 | – | 0.769 | – | <0.001 |
| Central​a | 2020 | 2 | 32.7 | 0.808 | 0.845 | 0.921 | 0.751 | – | 0.352 |
| Chattogram coast​a | 2024 | 1 | 54.2 | 0.649 | 0.793 | – | 0.604 | 0.678 | 0.181 |
| West-central​a | 2025 | 1 | 75.7 | 0.592 | 0.683 | – | 0.773 | 0.701 | 0.350 |

# Supporting Information

This file gives the pipeline settings with a map of the two hazard surfaces (Fig. S1), the construction of the composite risk and the sensitivity of its weights, the robustness checks of the comparison with the flood record, and the comparison of the graph network with tabular models and of the hazard surfaces with observed flooding, which the main text summarises or leaves out. Section, figure and table numbers without the prefix S refer to the main text. The code that produces every number here is at https://github.com/rakibhhridoy/rimes-mapathon, and the data at https://doi.org/10.5281/zenodo.23573843.

# S1 Pipeline and model settings

The pipeline runs as the chain of steps in Fig. S1.

## Infrastructure, terrain and features

Assets are fetched from the Overpass API by bounding box, in tiles of 0.75​∘, and clipped to the region afterwards. The boxes cross Bangladesh’s land border, so assets in India, or on land more than about 1 km outside the geoBoundaries border, are then dropped, keeping those on chars, coastal ground and offshore, and the risk grid and the grid-level validation are restricted the same way. Tiling avoids the timeouts that a query for a whole division produces. Three tag groups are requested, with lifeline assets covering hospitals, clinics, schools, colleges, shelters, bridges, embankments and dams, transport assets covering primary to trunk roads, railways and tagged bridges, and agricultural assets covering farmland, aquaculture, canals, ditches and marketplaces. Each feature is classified into one asset type from its tags and labelled with the district it falls in.

The SRTM tiles covering the bounding box are merged, reprojected to UTM zone 46N and clipped. From the projected elevation model the pipeline derives slope, the topographic wetness index (TWI), height above nearest drainage (HAND) and D8 flow accumulation. Drainage cells for HAND are those above the 90th percentile of flow accumulation, and HAND is the elevation difference between each cell and the nearest drainage cell found by a distance transform.

For each asset the pipeline samples the five terrain rasters at its representative point, samples WorldPop density, and computes the distance in metres to the nearest hospital, school, flood shelter, road and irrigation channel. Asset type enters as a categorical code, all features are standardised to zero mean and unit variance, and the label is the flood-prone status of the ground under the asset. Sampling reprojects the query coordinates into each raster’s coordinate system before reading it, and the pipeline stops if any feature comes out constant or if the labels contain a single class.

## Asset model, graph and calibration

The model that scores assets is a configuration setting, asset_model.type, and the default is class-weighted gradient boosting over the asset features, at 300 boosting iterations with a learning rate of 0.1. That default follows from the comparison in Section S7, which found ordinary tabular models matching or beating the graph network the system began with. The graph network remains available, as a two-layer GraphSAGE with 64 hidden channels and dropout of 0.3, trained with a class-weighted loss and early stopping on the calibration blocks, and so do a random forest and logistic regression. In the two riverine regions the scores shown on the map come from the same model given one more feature, the share of earlier Sentinel-1 events in which the ground flooded, trained to predict the flooding of the latest event year from the years before it and applied with the share over every event (Section 5.3). Every validation figure in this paper comes from the model without that feature, because the feature is built from the same events the models are scored against.

Assets also form the nodes of a k-nearest-neighbour graph with k=6, where any pair further apart than 5 km, measured in metres in the projected coordinate system, is left unconnected. Inverse-distance weights are stored with each edge, although the GraphSAGE mean aggregation does not use them, so every retained neighbour counts equally. Under the default model the edges carry nothing and the graph serves only to hold the features, labels and block assignment that every model shares.

Validation holds out whole spatial blocks of 10 km instead of random nodes, because neighbouring assets share terrain, so a random split leaves every validation node next to training nodes that carry almost the same features and label, and the resulting score measures memorisation rather than skill. Blocks are assigned at random with a fixed seed to three groups, with 80 % for training and the remainder split evenly between calibration and test. Early stopping and the isotonic calibration of the scores use the calibration blocks only, and every metric reported in this paper comes from the test blocks, which neither has seen. Isotonic regression returns the observed frequency of each level it fits, so a level holding a handful of calibration points that all flooded returns a probability of one. Each level is therefore shrunk by its Jeffreys estimate, (k+0.5)/(n+1) for k of n points flooded, which leaves a level carrying many points almost unchanged and pulls a level carrying few back towards the middle. In Rangpur and Rajshahi the raw scores give a Brier score of 0.069 on the test blocks and the calibrated probabilities 0.029, against 0.032 for always predicting the base rate, so the calibration corrects the inflated raw scores but adds little over the base rate. Flood prevalence differs from one set of blocks to another, which limits how far a calibration carries. In Rangpur and Rajshahi 6.4 % of the assets in the calibration blocks sit on flood-prone ground against 3.3 % in the test blocks, and the calibrated deciles nonetheless lie close to the diagonal there, while on the coast, with 5.5 % against 1.7 %, the calibration overstates the flooding in the test blocks (Fig. 3). The calibrated value is best read as an approximate probability for the region as a whole, reliable in its ranking but not in its level for any one area.

## Hazard surfaces

Two hazard surfaces are produced for every flood region, and the composite risk uses one of them according to the configuration setting risk.hazard_source. The first is ordinary Kriging of the model scores at the asset locations, with a spherical variogram fitted on up to 3,000 points and a residual Kriging pass that adds back the score residuals at the nodes. How much it can say between assets depends on the spatial structure of the scores, which the fitted variogram reports. The nugget is the share of the variance that is unexplained at zero distance, and the mean Kriging variance over the full sill is the share the interpolation leaves unexplained on average. On the current runs the nugget is 0.307 of the sill in Rangpur and Rajshahi and 0.742 in Sylhet, and the mean Kriging variance is 0.442 and 0.933 of the sill respectively, so in both regions the interpolated surface smooths substantially towards the regional average. In Rangpur and Rajshahi its standard deviation across the grid is 0.127 against 0.238 for the terrain surface described next, and it varies smoothly over tens of kilometres where the terrain surface follows individual channels and levees (Fig. S2).

The second surface is a class-weighted logistic regression from each grid cell’s own terrain, using elevation, slope, TWI, HAND and flow accumulation and nothing else, because distances to hospitals or roads describe exposure and vulnerability rather than the likelihood of flooding. It is trained at the asset locations on the same labels and the same spatial blocks as the asset model, and evaluated at every grid cell, so it varies at the resolution of the elevation model rather than at the spacing of mapped infrastructure. In Rangpur and Rajshahi it reaches an AUC of 0.621 on held-out blocks, and it is the default. The two surfaces can also be scored directly against observed flooding over every grid cell, not only at assets, and there the terrain surface reaches an AUC of 0.657 against 0.666 for the Kriged one. Both are written to the grid so the difference between them can be inspected. Because the terrain model is trained on the observed extents, it is a flood-probability surface fitted to what the radar saw. Were it trained on the proxy labels instead, the circularity described in Section 4.3 would apply with full force, and it would be no more than ground the ensemble calls flood-prone, smoothed into a probability.

# S2 Exposure, vulnerability, composite risk and aggregation

Exposure per cell is measured twice, because “where is infrastructure at risk” and “where would people be affected” are different questions with different answers. Infrastructure exposure is the type-weighted count of mapped assets in the cell, with hospitals and schools weighted at 0.20, bridges, roads and cropland at 0.15, shelters and embankments at 0.10 and other assets at 0.05, log-scaled to the unit interval. Population exposure is WorldPop density sampled at the cell centre, log-scaled the same way. Urban municipalities lead the first measure because that is where infrastructure is concentrated, while crowded floodplain settlements can lead the second.

Vulnerability combines population density with the distance from the cell centre to the nearest hospital, flood shelter and primary road, each normalised so that a greater distance means greater vulnerability. The configuration lists further components, a night-light proxy and an elderly-and-child ratio, that have no data source yet. Components without data are excluded rather than filled with a constant, and the remaining weights renormalised, and the weights actually applied are written to the output directory alongside the results. For the current runs the four applied components are population density (0.40), distance to hospital (0.27), distance to shelter (0.20) and distance to road (0.13).

Composite risk is the geometric mean of hazard, exposure and vulnerability, which keeps the multiplicative logic, in which no exposure means no risk, while staying on the unit scale of its inputs. Two composites are written, one for each exposure measure, and both are carried through to the administrative summaries. Because the composite is bounded by its weakest factor it rarely approaches 1 even where risk is locally worst, so fixed cut-offs would place almost every cell in the lowest class. Cells are therefore also assigned relative classes 1 to 5 by quintile of the non-zero composite, the usual practice for susceptibility maps. Class 0 marks cells with no mapped exposure, which is an absence of data, not an absence of risk.

Grid results are summarised for every union, upazila and district from geoBoundaries, and each summary carries the mean and maximum of both composites, counts of exposed assets by type, and a flag stating whether the unit holds any mapped assets at all, and units without mapped assets have their risk left undefined. In Rangpur and Rajshahi 1,199 of the 1,339 unions hold mapped assets and carry a score. Unit names repeat across Bangladesh, so each unit also carries a label naming its parent, such as “Abdullahpur (Gangachara)”. Hotspots are detected with the Getis–Ord Gi* statistic on k=8 nearest-neighbour weights, at 95 % confidence, over the cells that carry non-zero composite risk. Restricting the statistic to those cells keeps the weights matrix small enough to compute and avoids treating the empty majority of the grid as a sea of low values against which any occupied cell would look hot. Figure S3 maps the mean composite risk of every union in the three flood regions.

# S3 Sensitivity of the composite to its weights

The composite risk rests on weights that were chosen rather than measured, so the ranking it produces is recomputed under other defensible choices and compared with the published one by Spearman correlation and by the share of the top decile of cells that stays in the top decile. Weighting every asset type equally leaves the ordering almost unchanged, at ρ=0.942 in Rangpur and Rajshahi and ρ=0.908 on the coast, and so does weighting the vulnerability components equally. Doubling the exposure exponent in the geometric mean gives ρ=0.946 while retaining 0.829 of the top decile. Multiplying every chosen weight by a random factor between 0.5 and 1.5, over 20 draws, gives a mean ρ of 0.963 in Rangpur and Rajshahi, 0.956 in Sylhet and 0.978 on the coast, with 0.870 of the top decile retained.

Only one choice matters, reducing vulnerability to population density alone, which discards the distances to hospitals, shelters and roads and drops the correlation to 0.663 in Rangpur and Rajshahi and 0.425 on the coast, so the access distances rather than population drive the vulnerability term. The ordering of cells is therefore not an artefact of the particular weights, while the boundary of the top class is soft enough that a cell just inside it and a cell just outside should be treated alike.

# S4 Validation metrics

Every model in the chain is scored on spatial blocks held out from training, and the same block assignment is shared by the asset model and the terrain hazard model within a region. Metrics are the area under the ROC curve, which measures ranking, average precision, which is more sensitive to the rare positive class, and the Brier score for calibration. Average precision is also reported as a lift over the base rate, because an average precision of 0.07 against a positive rate of 0.01 is a sevenfold improvement over chance while the same value against a rate of 0.10 is worse than chance.

The validation step then measures on the held-out blocks whether model scores rank assets that flooded above those that did not. Whether the hazard surface agrees with observed flooding across the whole grid, and not only where assets happen to be, is measured by AUC and Spearman correlation over all cells, and, because that includes the ground the surface was fitted on, also by AUC over the cells of the test blocks alone, with the surface refitted for each of the repeated block assignments. And how closely the proxy labels match observed flooding is measured with the categorical scores used in flood mapping, the probability of detection, false alarm ratio and critical success index, which quantifies what the proxy-trained models were learning.

Every metric that compares two models or two label sources is repeated over several block assignments drawn with different seeds, because a single assignment gives one number with no sense of its spread, and differences are tested by the corrected resampled t-test of Nadeau & Bengio (2003), which allows for the assignments sharing the same assets. Each such comparison uses 20 assignments.

# S5 Robustness of the comparison with the flood record

The temporal test of the main text trains the asset model on the floods up to 2022 and scores it, and the record of earlier flooding, against the ground the 2024 floods reached, on held-out 10 km blocks over 20 assignments. Table S1 repeats it under seven changes, each aimed at one way the comparison could favour the record or the model. The validation region east of the Jamuna is checked the same way. Its national region shares 1,263 mapped assets with the Rangpur and Rajshahi box, which crosses the Jamuna, and 278 with the Sylhet box, and these are left out so that no asset is scored in both, leaving 3,874 of 5,415. Its October 2024 flood was a flash flood from the Meghalaya hills, and the first Sentinel-1 pass came five days after it began, so the fastest flooding in Sherpur may have drained unseen and the mask counts few flooded assets.

The first two checks leave the models alone and narrow the test set, because the record and the 2024 floods come from the same change detection, so ground that radar mistakes for water every monsoon, such as wet paddy, would count as flooded in both. Surfaces that stay dark in the dry season, such as sand bars and permanent wetland, are already rejected, since a pixel must darken against its dry-season baseline. The comparison is therefore repeated without the ground flagged in every earlier event, and without the ground the JRC record has ever seen under water, which removes seasonal wetland. Both removals also take away genuinely flood-prone ground, so they can only work against the record. In Sylhet the second leaves 72.0 % of assets, holding 29.9 % of the ground the 2024 floods reached.

The third check re-maps every event from Sentinel-1A alone, since Sentinel-1B failed in December 2021, so events before then were seen by two satellites and the 2024 events by one. Each event mask takes the lowest backscatter of its window, so a window with more scenes can map more flooding. With one satellite throughout, the earlier events shrank by up to 15.7 % in Rangpur and Rajshahi and 47.9 % on the coast, while in Sylhet Sentinel-1B had added no scenes and the masks are unchanged. The 2024 floods in Rangpur and Rajshahi covered 4.9–8.3 % of the region against 19.9–24.8 % for each earlier flood, and this difference is not a matter of scene counts, since the October 2024 and August 2017 windows hold the same number of Sentinel-1A scenes.

The fourth and fifth checks change what the model learns, since the published model learns a yes-or-no label, flooded in at least the region’s minimum number of earlier events, while the record keeps the fraction of earlier events in which the ground flooded. Because the 2024 floods are counted as flooded in any of them, the model is retrained on flooding in any earlier event, and, as a regression, on the fraction itself. On the coast the first of these is the published target, since one event already suffices there.

The last two widen the blocks, since the 10 km blocks are narrower than the fitted variogram ranges of the asset scores (main text, Section 5.5). With 40 km blocks in Sylhet, 15 of the assignments leave both classes in the test blocks.

# S6 Record length, skill over the record and planning lists

Three further analyses ask what the record offers a flood manager, using the same events and the same temporal split as the main text. The first rebuilds both the record and the model from every subset of k earlier events, for k from one to all of them, and scores each on the held-out 10 km blocks against the 2024 floods over 20 assignments, with the model’s label following the region’s rule capped at k. A record of one event can only say flooded or not, so it ranks coarsely, while the model still ranks every asset. Rebuilt from a single earlier flood, the record already led the model in Rangpur and Rajshahi, whereas in Sylhet it needed both earlier floods, and on the coast it trailed at every length (Table S2).

The second expresses each model’s AUC as skill over the record, (AUCmodel−AUCrecord)/(1−AUCrecord), the share of the record’s remaining ranking error that the model removes, in the way weather forecasts are scored against persistence. It is computed from the mean AUCs over the block assignments, because a ratio taken per assignment swings without bound where the record is nearly perfect. A score of zero means the model only matches the record, and a negative score that it falls behind.

The third turns the rankings into the list a planner would act on. Every asset is scored once out of fold, by models trained on the other nine of ten folds of 10 km blocks, so the whole region carries held-out scores. The record’s own list is every asset an earlier flood reached, 15.5 % of assets in Rangpur and Rajshahi, 20.8 % in Sylhet and 2.5 % on the coast, and the model and the model given the record each list the same number of their highest-scoring assets. Each list is scored by the share of the assets the 2024 floods reached that it holds. Facilities are hospitals, schools and flood shelters, and transport links are roads, bridges and railways. Facilities flooded in 2024 number only 11, 32 and 15 in the three regions, so their shares are indicative only. Ranking the out-of-fold scores within each fold rather than across the region did not change which list held more of the flooded assets in any region. On the coast the record’s lead holds for short lists only, since the top 5 % of assets it ranks held 38.6 % of the flooded assets against 16.0 % for the model, whereas the top 30 % held 54.4 % against 66.8 %, consistent with the model’s higher AUC over the region as a whole. The uncertainty of the out-of-fold comparison is estimated by resampling whole 10 km blocks 1,000 times, so that neighbouring assets stay together. The interval for the model’s AUC minus the record’s excludes zero in every study region, below it in the riverine regions and above it on the coast, and east of the Jamuna it reaches 0.019, so the lead there is probable rather than established.

Table S2. Record length, skill over the record and planning lists. Record-length AUCs are means over every subset of that many earlier events and over the block assignments; Sylhet has two events before 2022. Skill is the share of the record’s remaining ranking error that a model removes. The lists hold as many assets as an earlier flood reached in each region, with every asset scored out of fold.

|  | Rangpur & Rajshahi | Sylhet | South-west coast | Jamuna east |
| --- | --- | --- | --- | --- |
| AUC against the 2024 floods, record / model, by number of earlier events |  |  |  |  |
| One event | 0.812 / 0.768 | 0.726 / 0.794 | 0.579 / 0.699 | 0.839 / 0.751 |
| Two events | 0.873 / 0.788 | 0.851 / 0.797 | 0.634 / 0.794 | 0.868 / 0.718 |
| Three events | 0.896 / 0.801 | – | 0.668 / 0.807 | 0.881 / 0.713 |
| Skill over the record |  |  |  |  |
| Model | −0.905 | −0.364 | 0.416 | −1.416 |
| Model given the record | 0.276 | −0.016 | −0.063 | 0.325 |
| Share of assets flooded in 2024 held by each list (%), record / model / model given the record |  |  |  |  |
| All assets | 93.2 / 68.5 / 93.2 | 77.9 / 50.6 / 62.0 | 37.1 / 9.5 / 23.8 | 86.0 / 48.8 / 83.7 |
| Facilities | 36.4 / 18.2 / 45.5 | 40.6 / 3.1 / 6.2 | 80.0 / 0.0 / 73.3 | 50.0 / 50.0 / 50.0 |
| Transport links | 88.3 / 34.0 / 88.3 | 71.6 / 56.8 / 32.8 | 38.9 / 0.0 / 33.3 | 87.5 / 37.5 / 81.2 |
| Out-of-fold AUC, record / model / model given the record, and 95 % block-bootstrap interval of model minus record |  |  |  |  |
| AUC | 0.942 / 0.857 / 0.950 | 0.855 / 0.800 / 0.865 | 0.679 / 0.776 / 0.722 | 0.915 / 0.867 / 0.932 |
| Interval | −0.127 to −0.064 | −0.090 to −0.019 | 0.039 to 0.156 | −0.129 to 0.019 |
| Assets flooded in 2024 | 644 | 2,367 | 871 | 43 |

# S7 Graph network and tabular models

Graph neural networks, the design this system began with, learn from a node’s neighbours and, in the inductive form used here, generalise to nodes unseen in training (Hamilton et al., 2017). In hazard mapping they appeared first for landslides (Ma et al., 2025; X. Wang et al., 2023) and have since been applied to flood susceptibility on graphs of urban cells (Z. Wang et al., 2023) and of catchments linked by their topology (Liu et al., 2026), and a preprint reported a gain of 0.097 in AUC over the best tabular model on sub-watersheds linked by river connectivity (Sharma & Sharma, 2026). A review of 58 deep-learning flood studies found convolutional models usually the more accurate (Bentivoglio et al., 2022), and the searches made for this review found no graph model of the flood susceptibility of mapped infrastructure, nor one that links assets by proximity rather than by the flow of water.

A graph neural network is set against logistic regression, a random forest and gradient boosting on the same features, assets and blocks, with the wetness index alone as a model-free reference, over 20 assignments (Table S3, Fig. S4). Gradient boosting scores best in every region, and the graph network trails it measurably only in Rangpur and Rajshahi, by 0.028 in AUC (p=0.003), while the wetness index alone ranks close to chance, at 0.517, 0.535 and 0.577. Gradient boosting is therefore the default for every region, since choosing a model per region would fit the model to the split. Its AUC varies by 0.042 to 0.046 from one assignment to the next, so each single-assignment figure, including those in Table 5, is one draw from a distribution of that width.

Table S3. AUC on held-out test blocks, as mean ± standard deviation over 20 block assignments. Every model sees the same features at the same assets and the same blocks, so the only difference is the model. The last row is the paired difference between the graph network and whichever other model scores best in that region. Gradient boosting is the system’s default.

| Model | Rangpur & Rajshahi | Sylhet | South-west coast |
| --- | --- | --- | --- |
| Graph model (GraphSAGE) | 0.789 ± 0.047 | 0.854 ± 0.039 | 0.817 ± 0.057 |
| Gradient boosting | 0.817 ± 0.045 | 0.857 ± 0.042 | 0.839 ± 0.046 |
| Random forest | 0.816 ± 0.044 | 0.845 ± 0.052 | 0.838 ± 0.049 |
| Logistic regression | 0.786 ± 0.039 | 0.855 ± 0.036 | 0.809 ± 0.046 |
| Wetness index alone | 0.517 ± 0.030 | 0.535 ± 0.029 | 0.577 ± 0.041 |
| Graph minus best baseline | −0.028 | −0.003 | −0.022 |

On identical features and blocks the graph network trailed the best tabular model in every region, measurably only in Rangpur and Rajshahi. It linked each asset to its nearest neighbours, whereas the graphs reported to help elsewhere linked catchments along the flow of water (Liu et al., 2026; Sharma & Sharma, 2026), so the result bears only on proximity graphs of assets. Kriging fared no better, ranking observed flooding over the whole Sylhet grid at 0.544 against 0.822 for the terrain surface.

# S8 Hazard surfaces against observed flooding

On the coast the asset model still ranks flooded ground above dry ground (AUC 0.794), but both hazard surfaces are weak. Over the whole grid, training ground included, the terrain surface reaches 0.542 and the Kriged one 0.538, against 0.657 and 0.666 in Rangpur and Rajshahi and 0.822 and 0.544 in Sylhet. Refitted for each assignment and scored only on the cells of its test blocks, which hold only ground near mapped assets, the terrain surface reaches 0.656, 0.763 and 0.610, so the whole-grid figure overstated it in Sylhet. Surge flooding is governed by polder embankments, tidal channels and landfall position, none of which elevation describes.

# S9 Tests that share a region

The 15 rolling-origin tests come from 7 regions, and tests in one region share its assets, terrain and record, so the correlation between new ground and the record’s lead in the main text treats them as more independent than they are. Three checks take the region as the unit instead. With each region left out in turn, the Spearman correlation stayed between −0.857 and −0.671 (p=0.017 at the weakest), so no single region carries the relation. Resampling whole regions with replacement 5,000 times gave a 95 % interval for the correlation of −0.972 to −0.352, negative in 99.7 % of draws, while the interval for the share of new ground at which the record and the model are level widened to 2.6–42.6 %, which places the crossover less precisely than resampling tests does. Within each of the 5 regions with more than one test, the record’s lead was smaller on the test with the most new ground than on the test with the least in 5, so the relation also holds inside regions rather than only between them. A small flood could reach only ground that always floods, so the size of the flood, the share of assets it reached, was also held fixed. It was unrelated to new ground (ρ=−0.021), the relation between new ground and the record’s lead was unchanged with it held fixed (partial ρ=−0.785, p<0.001), and size added nothing once new ground was known (ρ=0.268, p=0.335).